# Fluidule MIDI Routing Design

## 1. System Overview

Fluidule is designed as a hybrid MIDI module using three main components:

• Raspberry Pi – MIDI routing and software synthesizer (FluidSynth)

• Arduino Pro Micro – DIN MIDI ↔ USB MIDI bridge

• Arduino Nano – User interface controller (LCD, encoders, buttons)

Raspberry Pi acts as the central MIDI router. All MIDI routing decisions are made there.

The Pro Micro only converts between DIN MIDI and USB MIDI.

The Nano handles the front panel UI and communicates with the Pi via USB serial.

## 2. Hardware MIDI Signal Paths

MIDI Inputs

• USB MIDI keyboard → Raspberry Pi USB

• DIN MIDI IN → Pro Micro → USB MIDI → Raspberry Pi

MIDI Outputs

• Internal audio → FluidSynth → USB DAC (e.g., Behringer UCA200)

• External MIDI → Pro Micro → DIN MIDI OUT

## 3. Core Routing Concept

All MIDI signals converge at the Raspberry Pi where ALSA MIDI performs routing.

USB Keyboard ─┐

├── Raspberry Pi (ALSA MIDI Router) ──→ FluidSynth

DIN MIDI IN ──┘                                └──→ DIN MIDI OUT

The Pi can dynamically connect or disconnect MIDI streams depending on selected routing mode.

## 4. Routing Modes

Mode A – Internal Sound

USB keyboard → FluidSynth

DIN MIDI IN → FluidSynth

Mode B – Soft Thru

USB keyboard → FluidSynth

USB keyboard → DIN MIDI OUT

DIN MIDI IN → FluidSynth

DIN MIDI IN → DIN MIDI OUT

Mode C – DIN Thru Only

USB keyboard → DIN MIDI OUT

DIN MIDI IN → DIN MIDI OUT

Mode D – Internal + Sequencer Out

USB keyboard → FluidSynth

DIN MIDI IN → FluidSynth

Sequencer/Player → FluidSynth

Sequencer/Player → DIN MIDI OUT

## 5. ALSA MIDI Implementation

Typical MIDI devices visible on Raspberry Pi:

Client A : USB MIDI Keyboard

Client B : Pro Micro MIDI Interface

Client C : FluidSynth

Example connections:

Internal Mode:

aconnect Keyboard:0 FluidSynth:0

aconnect ProMicro:0 FluidSynth:0

Soft Thru Mode:

aconnect Keyboard:0 FluidSynth:0

aconnect Keyboard:0 ProMicro:0

aconnect ProMicro:0 FluidSynth:0

aconnect ProMicro:0 ProMicro:0

## 6. UI Integration

Routing mode can be selected through the Fluidule front panel UI.

Example menu:

MIDI ROUTE

> Internal

SoftThru

DIN Thru

Seq + Out

The Arduino Nano sends the selected routing mode to the Raspberry Pi via USB serial.

## 7. System Responsibilities

Raspberry Pi

• MIDI routing

• FluidSynth sound engine

• Sequencer or MIDI file playback

Arduino Pro Micro

• DIN MIDI input

• DIN MIDI output

• USB MIDI device interface

Arduino Nano

• LCD display

• Encoders and buttons

• User interface logic

## 8. Design Advantages

• Clear separation of roles

• Flexible MIDI routing via ALSA

• Minimal firmware complexity in Pro Micro

• Easy future expansion (USB devices, external modules)

• Compatible with standard GM MIDI workflows